## Question Paper Set 3

Class XII

Time: 75 minutes | Max Marks: 35

### Section A: MCQs (7 x 1m = 7 Marks)

- The current density in a wire of radius 'r' and current 'I' is 'J'. If the radius is doubled and the current is halved, the new current density will be:

  - (a) J/2

  - (b) J/4

  - (c) J/8

  - (d) J/16

- The resistance of a carbon resistor is (47 +/- 4.7) kilo-ohms. The color of the third band is:

  - (a) Red

  - (b) Orange

  - (c) Yellow

  - (d) Violet

- Mobility of electrons is related to relaxation time (τ) as:

  - (a) µ is proportional to τ

  - (b) µ is proportional to 1/τ

  - (c) µ is proportional to τ²

  - (d) Independent of τ

- In a potentiometer experiment, the balancing length is 'L'. If a shunt resistance is connected across the test cell, the new balancing length L' will be:

  - (a) L' > L

  - (b) L' < L

  - (c) L' = L

  - (d) Zero

- Two resistors with temperature coefficients α₁ and α₂ are connected in series. The effective temperature coefficient is:

  - (a) α₁ + α₂

  - (b) (α₁ + α₂) / 2

  - (c) (R₁α₁ + R₂α₂) / (R₁ + R₂)

  - (d) √(α₁α₂)

- A wire of resistance 10 ohms is bent to form a complete circle. The resistance between any two diametrically opposite points is:

  - (a) 10 ohms

  - (b) 5 ohms

  - (c) 2.5 ohms

  - (d) 1.25 ohms

- The internal resistance of an ideal cell is:

  - (a) Zero

  - (b) 1 ohm

  - (c) Infinite

  - (d) Variable

### Section B: Short Answer (4 x 2m = 8 Marks)

- Numerical: A silver wire has a resistance of 2.1 ohms at 27.5 °C, and a resistance of 2.7 ohms at 100 °C. Determine the temperature coefficient of resistivity of silver.

- Conceptual: A steady current flows in a metallic conductor of non-uniform cross-section. Which of these quantities is constant along the conductor: current, current density, or drift speed?

- Definition: Define 'Terminal Potential Difference'. How does it differ from 'Emf'?

- Numerical: A cell of emf 2 V and internal resistance 0.1 ohm is connected to a 3.9 ohm resistor. What is the voltage across the terminals of the cell?

### Section C: Long Answer Type I (2 x 3m = 6 Marks)

- Derive the condition for the maximum current drawn from a combination of 'm' rows, each containing 'n' cells, connected to an external resistance R. (Mixed Grouping).

- State the two Kirchhoff’s rules. Explain the sign convention used while applying the loop rule.

### Section D: Case Study Based (1 x 4m = 4 Marks)

- Precision Measurements with Potentiometer: A potentiometer wire of length 10 m and resistance 20 ohms is connected in series with a 15-ohm resistor and a 5-V battery.

  - (i) Calculate the potential gradient along the wire. (2m)

  - (ii) If a cell gives a balance point at 600 cm, calculate the emf of the cell. (1m)

  - (iii) If the 15-ohm resistor is replaced by a 10-ohm resistor, will the balancing length increase or decrease? (1m)

### Section E: Long Answer Type II (2 x 5m = 10 Marks)

- (a) Derive an expression for the Conductivity (σ) of a material in terms of the number density of electrons (n) and relaxation time (τ). (3m) (b) A wire of resistance 8R is bent into the form of a square. Find the equivalent resistance between two opposite corners. (2m)

- (a) With the help of a circuit diagram, explain how a Meter Bridge is used to find the unknown resistance of a wire. (3m) (b) Why is it better to obtain the balance point near the 50 cm mark? (1m) (c) What are 'end errors' in a meter bridge? (1m)