### Section A: Multiple Choice Questions (40 questions each of 1 Mark)

- The SI unit of electric current is:

  - (a) Volt

  - (b) Ampere

  - (c) Ohm

  - (d) Watt

  - Answer: (b)

- The flow of 1 Coulomb of charge per second through a conductor is called:

  - (a) 1 Volt

  - (b) 1 Watt

  - (c) 1 Ampere

  - (d) 1 Ohm

  - Answer: (c) Logic: I = Q / t.

- Which of the following is the correct expression for Ohm's Law?

  - (a) V = I / R

  - (b) V = I * R

  - (c) I = V * R

  - (d) R = V * I

  - Answer: (b)

- The resistance of a conductor is inversely proportional to its:

  - (a) Length

  - (b) Area of cross-section

  - (c) Temperature

  - (d) Resistivity

  - Answer: (b) Logic: R = ρL / A.

- The drift velocity of electrons in a conductor is of the order of:

  - (a) 10⁸ m/s

  - (b) 10³ m/s

  - (c) 10⁻⁴ m/s

  - (d) 3 * 10¹⁰ m/s

  - Answer: (c) It is a very slow average speed despite high thermal speeds.

- The resistivity of a wire depends on:

  - (a) Length

  - (b) Area of cross-section

  - (c) Material

  - (d) All of the above

  - Answer: (c) Resistivity is an intrinsic property of the material.

- With the increase in temperature, the resistance of a semiconductor:

  - (a) Increases

  - (b) Decreases

  - (c) Remains constant

  - (d) Becomes zero

  - Answer: (b) Logic: More charge carriers are released at higher temperatures.

- Kirchhoff's First Law (Junction Rule) is a reflection of the conservation of:

  - (a) Energy

  - (b) Momentum

  - (c) Charge

  - (d) Mass

  - Answer: (c)

- Kirchhoff's Second Law (Loop Rule) is based on the conservation of:

  - (a) Charge

  - (b) Energy

  - (c) Momentum

  - (d) Mass

  - Answer: (b)

- The internal resistance of a cell depends on:

  - (a) Distance between electrodes

  - (b) Nature of electrolyte

  - (c) Area of electrodes

  - (d) All of the above

  - Answer: (d)

- Two resistances R1 and R2 are connected in parallel. Their equivalent resistance is:

  - (a) R1 + R2

  - (b) (R1 + R2) / R1R2

  - (c) R1R2 / (R1 + R2)

  - (d) √(R1R2)

  - Answer: (c)

- The SI unit of electrical conductivity is:

  - (a) Ohm-meter

  - (b) Ohm / meter

  - (c) Mho / meter (or Siemens / meter)

  - (d) Mho-meter

  - Answer: (c)

- A wire of resistance R is stretched to double its length. Its new resistance will be:

  - (a) 2R

  - (b) 4R

  - (c) R/2

  - (d) R/4

  - Answer: (b) Logic: When length doubles, area halves to keep volume constant. R' = ρ(2L)/(A/2) = 4R.

- The potential gradient along a potentiometer wire should be:

  - (a) As large as possible

  - (b) As small as possible

  - (c) Zero

  - (d) Variable

  - Answer: (b) A smaller gradient increases the sensitivity of the potentiometer.

- In a Wheatstone bridge, the bridge is balanced when the current through the galvanometer is:

  - (a) Maximum

  - (b) Minimum

  - (c) Zero

  - (d) Half of the total current

  - Answer: (c)

- The color code for a resistor with 5% tolerance is:

  - (a) Gold

  - (b) Silver

  - (c) Red

  - (d) Brown

  - Answer: (a)

- Which material is used for the heater element in an electric iron?

  - (a) Copper

  - (b) Iron

  - (c) Nichrome

  - (d) Tungsten

  - Answer: (c) Because of its high resistivity and high melting point.

- The terminal potential difference (V) of a cell of emf (E) and internal resistance (r) during discharging is:

  - (a) V = E + Ir

  - (b) V = E - Ir

  - (c) V = Ir

  - (d) V = E

  - Answer: (b)

- If n identical cells of emf E and internal resistance r are connected in series, the total emf and internal resistance are:

  - (a) nE, nr

  - (b) E, r/n

  - (c) nE, r/n

  - (d) E, nr

  - Answer: (a)

- [Logic] When cells are connected in parallel, it helps to increase the:

  - (a) Voltage

  - (b) Current

  - (c) Internal resistance

  - (d) Lifetime of the circuit

  - Answer: (b) Parallel connection provides more current capacity.

- A current of 2 A flows through a conductor of resistance 10 Ohms for 1 minute. The heat produced is:

  - (a) 20 J

  - (b) 200 J

  - (c) 1200 J

  - (d) 2400 J

  - Answer: (d) Logic: H = I²Rt = (2)² * 10 * 60 = 2400 J.

- The relaxation time of electrons in a metal:

  - (a) Increases with increase in temperature

  - (b) Decreases with increase in temperature

  - (c) Does not depend on temperature

  - (d) Is always constant

  - Answer: (b) Higher temperature leads to more frequent collisions.

- The device used for measuring the internal resistance of a cell is:

  - (a) Voltmeter

  - (b) Ammeter

  - (c) Potentiometer

  - (d) Galvanometer

  - Answer: (c)

- In the meter bridge, the null point is usually preferred at the middle of the wire to:

  - (a) Increase resistance

  - (b) Decrease current

  - (c) Minimize the percentage error

  - (d) Make it easy to read

  - Answer: (c)

- The power dissipated in a resistor is given by:

  - (a) VI

  - (b) I²R

  - (c) V² / R

  - (d) All of these

  - Answer: (d)

- Which of the following is a non-ohmic conductor?

  - (a) Copper wire

  - (b) Aluminum wire

  - (c) Semiconductor diode

  - (d) Silver wire

  - Answer: (c) V is not proportional to I for diodes.

- The mobility of charge carriers is defined as:

  - (a) Drift velocity per unit electric field

  - (b) Electric field per unit drift velocity

  - (c) Current per unit area

  - (d) Relaxation time per unit mass

  - Answer: (a) Logic: μ = Vd / E.

- One kilowatt-hour (1 kWh) is equal to:

  - (a) 3.6 * 10³ J

  - (b) 3.6 * 10⁴ J

  - (c) 3.6 * 10⁵ J

  - (d) 3.6 * 10⁶ J

  - Answer: (d)

- When a current is passed through a junction of two different metals, heat is evolved or absorbed. This is called:

  - (a) Joule effect

  - (b) Peltier effect

  - (c) Seebeck effect

  - (d) Thomson effect

  - Answer: (b)

- For which of the following materials is the temperature coefficient of resistance negative?

  - (a) Copper

  - (b) Carbon

  - (c) Iron

  - (d) Silver

  - Answer: (b) Carbon is a semiconductor; its resistance decreases with heat.

- The resistance of an ideal voltmeter is:

  - (a) Zero

  - (b) Very low

  - (c) Very high

  - (d) Infinite

  - Answer: (d)

- The resistance of an ideal ammeter is:

  - (a) Zero

  - (b) Very low

  - (c) Very high

  - (d) Infinite

  - Answer: (a)

- Two bulbs are marked 25 W - 220 V and 100 W - 220 V. Which bulb has higher resistance?

  - (a) 25 W bulb

  - (b) 100 W bulb

  - (c) Both have same resistance

  - (d) Cannot be determined

  - Answer: (a) Logic: R = V² / P. Lower power means higher resistance.

- If n identical resistors are first connected in series and then in parallel, the ratio of equivalent resistances Rs / Rp is:

  - (a) n

  - (b) 1 / n

  - (c) n²

  - (d) 1 / n²

  - Answer: (c) Rs = nR, Rp = R/n. Rs/Rp = n².

- The fuse wire should have:

  - (a) High resistance and high melting point

  - (b) Low resistance and high melting point

  - (c) High resistance and low melting point

  - (d) Low resistance and low melting point

  - Answer: (c)

- The current density is a:

  - (a) Scalar quantity

  - (b) Vector quantity

  - (c) Dimensionless quantity

  - (d) None of these

  - Answer: (b) It has both magnitude and a specific direction of flow.

- The local neutral point in a potentiometer means:

  - (a) Current in the primary circuit is zero

  - (b) Current in the secondary circuit is zero

  - (c) Potential difference is infinite

  - (d) The wire has broken

  - Answer: (b)

- The copper wires are used as connecting wires because:

  - (a) They have high electrical conductivity

  - (b) They are cheap

  - (c) They are ductile

  - (d) All of the above

  - Answer: (d)

- Superconductors are materials that have:

  - (a) Very high resistance

  - (b) Zero resistance

  - (c) Negative resistance

  - (d) Variable resistance

  - Answer: (b) This happens below a critical temperature.

- The potential difference across a conductor is doubled. The drift velocity of electrons will:

  - (a) Be halved

  - (b) Be doubled

  - (c) Remain same

  - (d) Become four times

  - Answer: (b) Logic: Vd = (eEτ / m) and E = V/L. So, Vd ∝ V.

### Section B: Short Answer Questions (30 questions each of 2 Marks)

1. Define electric current. Is it a scalar or a vector quantity?

- Answer: Electric current is the rate of flow of electric charge through a cross-section of a conductor (I = Q / t).

- Logic: It is a scalar quantity. While it has a specified direction of flow, it does not follow vector addition laws; it follows simple algebraic addition.

2. State Ohm's Law and write its mathematical expression.

- Answer: Ohm's Law states that the current flowing through a conductor is directly proportional to the potential difference across its ends, provided physical conditions like temperature remain constant.

- Formula: V = IR, where R is the resistance.

3. Define 'Drift Velocity' of electrons.

- Answer: It is the average velocity with which free electrons in a conductor get drifted toward the positive terminal under the influence of an applied external electric field.

- Value: It is typically very small, around 10⁻⁴ m/s.

4. What is the relation between current (I) and drift velocity (vd)?

- Answer: I = nAevd

- Terms: n = free electron density, A = area of cross-section, e = electron charge, vd = drift velocity.

5. Define 'Mobility' of a charge carrier. State its SI unit.

- Answer: Mobility (µ) is the magnitude of drift velocity per unit electric field.

- Formula: µ = vd / E. SI Unit: m² / (V s).

6. How does the resistivity of (i) a metal and (ii) a semiconductor change with a rise in temperature?

- Answer: 1. Metal: Resistivity increases with temperature because collisions become more frequent. 2. Semiconductor: Resistivity decreases with temperature because more charge carriers are liberated.

7. Define 'Relaxation Time' (τ).

- Answer: It is the average time interval between two successive collisions of a free electron with the ions or atoms in a conductor.

8. State Kirchhoff's First Law (Junction Rule). What is it based on?

- Answer: The algebraic sum of currents meeting at any junction in an electrical circuit is zero (Sum of I = 0).

- Basis: It is based on the Law of Conservation of Charge.

9. State Kirchhoff's Second Law (Loop Rule). What is it based on?

- Answer: The algebraic sum of changes in potential around any closed loop in a circuit is zero.

- Basis: It is based on the Law of Conservation of Energy.

10. What is a Wheatstone Bridge? Write the condition for a balanced bridge.

- Answer: It is an arrangement of four resistances used to find an unknown resistance accurately.

- Condition: When no current flows through the galvanometer, P / Q = R / S.

Shutterstock

11. Why is a potentiometer preferred over a voltmeter for measuring the emf of a cell?

- Answer: A potentiometer measures emf in an "open circuit" (drawing no current), providing an accurate reading. A voltmeter draws some current, measuring a value slightly lower than the actual emf.

12. Define the 'Potential Gradient' of a potentiometer wire.

- Answer: It is the potential drop per unit length of the potentiometer wire.

- Formula: k = V / L. SI Unit: V/m.

13. What are the factors affecting the internal resistance of a cell?

- Answer: 1. Distance between electrodes. 2. Concentration of electrolyte. 3. Area of electrodes immersed. 4. Temperature of the electrolyte.

14. Write the relation between emf (E), terminal potential difference (V), and internal resistance (r).

- Answer: V = E - Ir (during discharge).

15. Two wires of equal length, one of copper and the other of manganin, have the same resistance. Which wire is thicker?

- Answer: Manganin is thicker.

- Logic: Resistance R = ρL / A. Since Manganin has higher resistivity (ρ), it must have a larger area (A) to maintain the same resistance as copper.

16. Define current density. Is it a vector or scalar?

- Answer: Current density (J) is the current flowing per unit area of cross-section held normal to the direction of flow. J = I / A.

- Logic: It is a vector quantity.

17. What is 'Electrical Conductivity'? State its SI unit.

- Answer: It is the reciprocal of electrical resistivity. σ = 1 / ρ.

- SI Unit: Siemens/meter (S/m) or 1 / (ohm m).

18. Why are connecting wires in a bridge made of thick copper strips?

- Answer: Thick strips have negligible resistance, ensuring that the measured values represent only the resistors in the circuit and not the wiring itself.

19. Define the term 'Temperature Coefficient of Resistivity' (α).

- Answer: It is the fractional change in resistivity per unit change in temperature.

20. What is an 'Ohmic' conductor? Give an example.

- Answer: A conductor that obeys Ohm's law, where the V-I graph is a straight line.

- Example: Most metals like Copper and Silver.

21. What happens to the drift velocity if the length of the conductor is doubled, keeping the potential difference constant?

- Answer: The drift velocity is halved.

- Logic: vd = (e V τ) / (m L). Since vd is inversely proportional to L, doubling L halves vd.

22. Distinguish between emf and terminal potential difference.

- Answer: Emf is the voltage of a cell when no current is flowing. Terminal potential is the voltage when the cell is part of a closed circuit.

23. Under what condition is the terminal potential difference of a cell greater than its emf?

- Answer: During the charging of a cell. In this case, V = E + Ir.

24. State the principle of a Potentiometer.

- Answer: The potential drop across any length of a uniform wire is directly proportional to that length, provided a constant current flows through it (V is proportional to L).

25. Why is the meter bridge wire usually made of alloys like Constantan?

- Answer: Alloys have high resistivity and a very low temperature coefficient, meaning their resistance doesn't change even if they heat up slightly.

26. How can you increase the sensitivity of a potentiometer?

- Answer: By reducing the potential gradient, either by increasing the length of the wire or by reducing the current in the primary circuit.

27. What is 'Superconductivity'?

- Answer: The phenomenon where a material's electrical resistance drops to exactly zero when cooled below a specific critical temperature.

28. A wire of resistance 5 ohms is stretched so its length is doubled. What is the new resistance?

- Answer: 20 ohms.

- Logic: Doubling length (L) halves the area (A). Since R = ρL/A, the new resistance becomes 4 times the original (2 * 2 = 4).

29. What is the equivalent resistance of n identical resistors in (i) series and (ii) parallel?

- Answer: 1. Series: Rs = nR. 2. Parallel: Rp = R / n.

30. Define electrical power. Write its three mathematical forms.

- Answer: It is the rate at which electrical energy is consumed in a circuit.

- Forms: P = VI, P = I²R, and P = V² / R.

### Section C: Categorized Numerical Gallery

### Level 1: Direct Application (Single Formula & SI Units)

1. Drift Velocity Calculation: A current of 1.6 A flows through a copper wire of cross-sectional area 1 mm². If the free electron density is 8 * 10²⁸ per m³, calculate the drift velocity.

- Trap: Convert mm² to m². 1 mm² = 10⁻⁶ m².

- Solution: vd = I / (nAe) vd = 1.6 / (8 * 10²⁸ * 10⁻⁶ * 1.6 * 10⁻¹⁹) vd = 1.6 / (12.8 * 10³) = 1.6 / 12800 vd = 1.25 * 10⁻⁴ m/s.

2. Resistance and Resistivity: Calculate the resistance of a copper wire of length 2 m and area of cross-section 1.7 * 10⁻⁶ m². (Resistivity of copper = 1.7 * 10⁻⁸ ohm m).

- Solution: R = ρL / A R = (1.7 * 10⁻⁸ * 2) / (1.7 * 10⁻⁶) R = 2 * 10⁻² = 0.02 ohms.

3. Ohm's Law and Power: A heating element is marked 210 V, 630 W. Calculate the current drawn by the element and its resistance.

- Solution: I = P / V = 630 / 210 = 3 A. R = V / I = 210 / 3 = 70 ohms.

### Level 2: Multi-Formula (Combining Concepts)

4. Temperature Effect on Resistance: The resistance of a silver wire is 2.1 ohms at 27.5 °C and 2.7 ohms at 100 °C. Determine the temperature coefficient of resistivity (α) of silver.

- Formula: α = (R2 - R1) / [R1(T2 - T1)]

- Solution: α = (2.7 - 2.1) / [2.1 * (100 - 27.5)] α = 0.6 / (2.1 * 72.5) = 0.6 / 152.25 α = 0.0039 per °C.

5. Internal Resistance of a Cell: A battery of emf 10 V and internal resistance 3 ohms is connected to a resistor. If the current in the circuit is 0.5 A, what is the resistance of the resistor? What is the terminal voltage of the battery?

- Solution (Part A): I = E / (R + r) => 0.5 = 10 / (R + 3) 0.5R + 1.5 = 10 => 0.5R = 8.5 => R = 17 ohms.

- Solution (Part B): V = E - Ir = 10 - (0.5 * 3) = 10 - 1.5 = 8.5 V.

6. Combinations of Cells: Six cells, each of emf 2 V and internal resistance 0.5 ohms, are connected in series to an external resistor of 9 ohms. Calculate the current in the circuit.

- Concept: Total E = nE; Total r = nr.

- Solution: Total E = 6 * 2 = 12 V; Total r = 6 * 0.5 = 3 ohms. I = Total E / (R + Total r) = 12 / (9 + 3) = 12 / 12 = 1 A.

### Level 3: Conceptual Application (HOTS)

7. Balanced Wheatstone Bridge: In a Wheatstone bridge, the four arms have resistances P = 10 ohms, Q = 100 ohms, R = 40 ohms, and S = X. Find the value of X if the bridge is balanced.

- Condition: P / Q = R / S

- Solution: 10 / 100 = 40 / X 1 / 10 = 40 / X => X = 400 ohms.

8. Potentiometer Comparison: In a potentiometer, a cell of emf E1 gives a balance point at 40 cm length of wire. When this cell is replaced by another cell of emf E2, the balance point shifts to 60 cm. Find the ratio E1 / E2.

- Principle: E ∝ L

- Solution: E1 / E2 = L1 / L2 = 40 / 60 = 2 : 3.

9. Meter Bridge Error: In a meter bridge, the null point is found at a distance of 33.7 cm from A. If now a resistance of 12 ohms is connected in parallel with S, the null point occurs at 51.9 cm. Determine the values of R and S.

- Logic: Use the formula R / S = L / (100 - L) for both cases and solve the simultaneous equations.

- Solution: Case 1: R / S = 33.7 / 66.3. Case 2: R / S_equivalent = 51.9 / 48.1 where 1/S_eq = 1/S + 1/12. Solving gives R = 6.8 ohms and S = 13.4 ohms.

10. Stretching of Wire (Volume Conservation): A wire of resistance R is stretched such that its radius decreases by n%. Find the new resistance.

- Trap: Radius change affects Area as square (A ∝ r²).

- Concept: Volume is constant. If radius decreases by n%, Area decreases by approx 2n%, and Length increases to compensate.

- General Result: If radius becomes r / k, resistance becomes k⁴ * R.

### Section D: Long Answer Questions (15 questions each of 4 marks)

1. Derive an expression for the drift velocity of free electrons in a conductor in terms of relaxation time.

- Step 1: Acceleration. When an electric field E is applied, the force on an electron is F = -eE. Acceleration a = F / m = -eE / m.

- Step 2: Velocity Equation. Using the first equation of motion, v = u + at. Since the average initial thermal velocity u is zero, vd = a * τ.

- Step 3: Substitution. Substitute the value of a into the velocity equation.

- Result: vd = - (eE / m) * τ.

- Marking Markers: Force/Acceleration (1m), Concept of Relaxation Time (1m), Substitution (1m), Final Formula (1m).

2. Deduce Ohm's Law from the expression for drift velocity.

- Step 1: Current Relation. Use the formula I = nAevd.

- Step 2: vd Substitution. Substitute vd = (eVτ) / (mL) (since E = V/L).

- Step 3: Rearrangement. I = nAe * (eVτ / mL) = (ne²Aτ / mL) * V.

- Step 4: Comparison. Write V / I = (mL) / (ne²Aτ).

- Result: V / I = R, where R = (m / ne²τ) * (L / A).

- Marking Markers: vd formula (1m), Substitution (1m), Isolating V/I (1m), Identifying R (1m).

3. State Kirchhoff’s Laws for electrical networks. Using these laws, derive the balance condition for a Wheatstone Bridge.

- Step 1: Statements. State the Junction Rule (Charge conservation) and Loop Rule (Energy conservation).

- Step 2: Balanced State. Define the state where current through the galvanometer Ig = 0.

- Step 3: Application. Apply the Loop Rule to loops ABDA and BCDB.

- Step 4: Ratios. Equate the potential drops.

- Result: P / Q = R / S.

4. Describe the principle and working of a Potentiometer. How can it be used to compare the emf of two primary cells?

- Step 1: Principle. Potential drop is proportional to length (V ∝ L).

- Step 2: Circuit. Describe the primary circuit (constant current) and secondary circuit (test cells).

- Step 3: Balancing. Find null points L1 for cell E1 and L2 for cell E2.

- Result: E1 / E2 = L1 / L2.

5. How is a Potentiometer used to determine the internal resistance of a cell? Derive the necessary formula.

- Step 1: Setup. Connect a cell in the secondary circuit with a resistance box R and a key.

- Step 2: Open Circuit. With key open, find balancing length L1 (measures Emf E).

- Step 3: Closed Circuit. With key closed, find balancing length L2 (measures Terminal Voltage V).

- Step 4: Formula. Use r = R * [(E/V) - 1].

- Result: r = R * [(L1/L2) - 1].

6. State the principle of a Meter Bridge. Explain its use in determining the unknown resistance of a wire.

- Step 1: Principle. It is based on the balanced Wheatstone bridge.

- Step 2: Construction. A 1m wire, a resistance box R, and an unknown resistance S.

- Step 3: Working. Move the jockey to find the null point at length L.

- Result: S = R * [(100 - L) / L].

7. Derive an expression for the equivalent emf and internal resistance of two cells connected in Series.

- Step 1: Setup. Two cells E1, r1 and E2, r2 connected end-to-end.

- Step 2: Potential Difference. V = VA - VC = (VA - VB) + (VB - VC).

- Step 3: Substitution. V = (E1 - Ir1) + (E2 - Ir2).

- Step 4: Combination. V = (E1 + E2) - I(r1 + r2).

- Result: E_eq = E1 + E2 and r_eq = r1 + r2.

8. Derive an expression for the equivalent emf and internal resistance of two cells connected in Parallel.

- Step 1: Setup. Two cells connected across the same two points.

- Step 2: Current. I = I1 + I2.

- Step 3: Expressing I. I1 = (E1 - V) / r1 and I2 = (E2 - V) / r2.

- Step 4: Rearranging. Combine and solve for V.

- Result: E_eq = (E1r2 + E2r1) / (r1 + r2) and r_eq = (r1r2) / (r1 + r2).

9. Define 'Mobility' of charge carriers. Derive the relation between mobility and electrical conductivity.

- Step 1: Definitions. μ = vd / E and J = σE.

- Step 2: Current Density. J = nevd.

- Step 3: Substitution. σE = ne(μE).

- Result: σ = neμ.

10. Discuss the variation of resistivity with temperature for (i) Metals, (ii) Alloys, and (iii) Semiconductors. Draw the respective graphs.

- Metals: ρ increases linearly with T.

- Aloys (Manganin): ρ is nearly independent of T.

- Semiconductors: ρ decreases exponentially as T increases.

11. Define 'Current Density' and 'Electrical Conductivity'. Derive the vector form of Ohm’s Law.

- Step 1: Setup. Start with V = IR.

- Step 2: Substitution. EL = (JA) * (ρL / A).

- Step 3: Cancellation. E = Jρ.

- Result: J = σE.

12. Explain the 'Combination of Resistors' in series and parallel. Derive the equivalent resistance for three resistors in each case.

- Series: Current is same. V = V1 + V2 + V3. Result: Rs = R1 + R2 + R3.

- Parallel: Potential is same. I = I1 + I2 + I3. Result: 1/Rp = 1/R1 + 1/R2 + 1/R3.

13. A wire of length L and resistance R is stretched to n times its original length. Derive the new resistance and resistivity.

- Volume Conservation: A*L = A' * (nL), so A' = A / n.

- Resistance: R' = ρ(nL) / (A/n) = n² * (ρL / A).

- Result: R' = n²R. Resistivity remains unchanged as it is a material property.

14. What are 'Non-Ohmic' devices? Explain with examples of a vacuum diode and a semiconductor diode.

- Definition: Devices where V is not proportional to I.

- Graphs: Describe the non-linear V-I characteristics.

15. Describe the use of a Potentiometer to determine the internal resistance of a cell with a clear circuit diagram.(This provides more detail than Q5, focusing on the experiment protocol).

- Points: Primary circuit setup, high resistance protection, null point detection, and the calculation using r = R(L1/L2 - 1).

### Section E: Case Studies  ( each of 4 marks)

### Case Study 1: Household Electric Circuits

In a typical home, all electrical appliances are connected in parallel to the main supply. This ensures that each appliance receives the same voltage (usually 220 V) and can be operated independently. A fuse or circuit breaker is connected in series with the live wire to protect the circuit from overloading or short-circuiting.

- Why are household appliances connected in parallel rather than in series? (1m)

  - Answer: In parallel, each appliance receives the full supply voltage, and if one appliance fuses or is switched off, the others continue to work.

- A 1000 W electric heater is used on a 220 V line. Calculate the current drawn. (1m)

  - Solution: I = P / V = 1000 / 220 = 4.55 A.

- If a 5 A fuse is used in this circuit, will it blow if a 500 W toaster is turned on simultaneously with the heater?(2m)

  - Solution: Total Power = 1000 + 500 = 1500 W. Total Current = 1500 / 220 = 6.82 A. Since 6.82 A > 5 A, the fuse will blow.

### Case Study 2: The Car Battery and Internal Resistance

A car battery (lead-acid accumulator) must provide a very high current (about 100 A to 200 A) for a few seconds to start the engine. Even though the emf of the battery is only 12 V, it can deliver such high current because its internal resistance is extremely low. As a battery ages, its internal resistance increases.

- Why does the terminal voltage of a battery drop significantly when the starter motor is turned on? (1m)

  - Answer: Due to the high current (I), the internal voltage drop (Ir) becomes significant, reducing the terminal voltage (V = E - Ir).

- A battery has an emf of 12 V and internal resistance of 0.05 ohms. What is the maximum current it can draw if short-circuited? (1m)

  - Solution: I_max = E / r = 12 / 0.05 = 240 A.

- If the headlights of a car dim when the engine starts, what does it indicate about the battery? (2m)

  - Answer: It indicates that the battery has a high internal resistance (likely due to aging), causing a large voltage drop when the high-current starter motor is engaged.

### Case Study 3: The Potentiometer - A Precise Tool

In laboratories, a potentiometer is used to measure emf and internal resistance. Unlike a voltmeter, which has a finite resistance and draws some current from the source, a potentiometer works on the "null deflection" method. At the balance point, no current is drawn from the cell under test.

- Why is the potentiometer called an "ideal voltmeter"? (1m)

  - Answer: Because at the balance point, it draws zero current from the source, effectively acting as a voltmeter with infinite resistance.

- What happens to the balancing length if the current in the primary circuit is increased? (1m)

  - Answer: The potential gradient (k) increases, so the balancing length (L = E/k) decreases.

- Can a potentiometer measure a potential difference larger than the emf of its own driver cell? (2m)

  - Answer: No. The potential drop across the potentiometer wire cannot exceed the emf of the driver cell. If the test voltage is higher, a null point will never be reached on the wire.

### Case Study 4: Temperature Sensors (Resistance Thermometers)

The resistance of metals changes linearly with temperature. This property is used in Platinum Resistance Thermometers (PRTs). By measuring the resistance of a platinum wire, the temperature of an environment can be calculated very accurately.

- Write the formula relating resistance (R) to temperature (T). (1m)

  - Answer: R = R0 [1 + α(T - T0)], where α is the temperature coefficient of resistance.

- For most metals, is the value of 'α' positive or negative? (1m)

  - Answer: Positive (Resistance increases with temperature).

- A platinum wire has a resistance of 5 ohms at 0 °C and 5.23 ohms at 100 °C. Calculate the temperature when the resistance is 5.795 ohms. (2m)

  - Solution: Using (R_t - R_0) / (R_100 - R_0) = (T - 0) / (100 - 0) (5.795 - 5) / (5.23 - 5) = T / 100 => 0.795 / 0.23 = T / 100 T = 345.65 °C.

### Case Study 5: Safety and the Earth Wire

Electrical safety in appliances with metallic bodies (like refrigerators or toasters) is ensured by the third pin—the "Earth" wire. If a fault occurs and the live wire touches the metallic body, the earth wire provides a low-resistance path for the current to flow into the ground, causing the fuse to blow and preventing electric shock.

- What is the function of the earth wire in a three-pin plug? (1m)

  - Answer: It provides a low-resistance safety path to the ground for any leakage current from the metallic body of an appliance.

- Why is the earth pin in a plug longer and thicker than the live and neutral pins? (1m)

  - Answer: It is longer so that it connects to the ground first (for safety) and thicker so it cannot be accidentally inserted into the live or neutral holes.

- How does earthing cause a fuse to blow during a short circuit to the appliance body? (2m)

  - Answer: When the live wire touches the body, the low-resistance earth path causes a massive surge in current. This current exceeds the fuse rating, melting the fuse and breaking the circuit.