## Question Paper Set 1 - Keys

Class XII

Time: 75 minutes | Max Marks: 35

### Section A

- (c) Momentum (Direction changes, so velocity changes, even though speed is constant).

- (c) 4B (B is proportional to I/r. 2 / 0.5 = 4).

- (d) Electron (Mass is too small; relativistic mass increase happens too quickly).

- (b) Repulsive (Same direction = Attract; Opposite = Repel).

- (b) Low resistance in parallel.

- (d) Magnetic field strength (m = NIA).

- (b) (1/2) μ₀nI.

### Section B

- B = (μ₀NI) / 2R = (4π * 10⁻⁷ * 100 * 0.5) / (2 * 0.1) = (2π * 10⁻⁵) / 0.2 = 3.14 * 10⁻⁴ T.

- Because the magnetic force is always perpendicular to the velocity (displacement). Since W = F.d cos(90), work done is zero.

- The magnetic moment of an electron in the first orbit of H-atom. 9.27 * 10⁻²⁴ A m².

- S = (Ig * G) / (I - Ig) = (0.001 * 100) / (1 - 0.001) ≈ 0.1 / 1 = 0.1 ohm.

### Section C

- dB = (μ₀/4π) * (I dl sinθ / r²). For center, θ=90, r=R. Integrate dl from 0 to 2πR to get B = μ₀I/2R.

- τ = Force * lever arm = (BIl) * (b sinθ) = NIAB sinθ.

### Section D

- (i) F = q [E + (v x B)]. (ii) v = E / B. (iii) Yes. The condition v = E/B is independent of the mass and charge of the particle, as long as the forces are in opposite directions.

### Section E

- (a) Integral B.dl = μ₀I. (b) For rectangular loop, BL = μ₀(nL*I), so B = μ₀nI. (c) Field increases significantly.

- (a) B₁ = μ₀I₁ / 2πd. F = B₁ I₂ L. F/L = (μ₀ I₁ I₂) / 2πd. (b) Standard definition of 1 Ampere.