## Question Paper Set 3 - Keys

Class XII

Time: 75 minutes | Max Marks: 35

### Section A

- (a) Upwards (Apply Right-Hand Thumb Rule for B-field of wire, then Fleming's Left-Hand Rule for the electron).

- (c) 2B (B is proportional to 1/r; if r is halved, B is doubled).

- (d) Voltmeter of range 0-10 V (R = G + V/Ig; higher range requires higher series resistance).

- (b) 2m (m = I * A = I * πr²; 0.5 * 2² = 2).

- (c) Zero (Field exists only within the core of the toroid).

- (d) Velocity (T = 2πm / qB).

- (a) S (S is proportional to N * B; 2 * 0.5 = 1).

### Section B

- B = μ₀I / 2πr = (4π * 10⁻⁷ * 35) / (2π * 0.2) = (2 * 10⁻⁷ * 35) / 0.2 = 3.5 * 10⁻⁵ T.

- Wire 1 produces a magnetic field at Wire 2. Applying Fleming's Left Hand Rule, the force on Wire 2 is directed towards Wire 1. The same happens in reverse, leading to attraction.

- It is the current required to produce a deflection of one scale division. k = 1 / Si (Reciprocal of Current Sensitivity).

- F = qvB sinθ = (1.6 * 10⁻¹⁹) * (2 * 10⁶) * 0.5 * sin(30°) = 1.6 * 10⁻¹³ * 0.5 = 8 * 10⁻¹⁴ N.

### Section C

- Consider  from a current element . Vertical components cancel. Integrate  over the loop to get B = (μ₀ I R²) / [2(R² + x²)^(3/2)].

- Solenoid is a straight coil; Toroid is a circular "doughnut" shaped coil. Both have B = μ₀nI (Inside).

### Section D

- (i) Vertically Upwards (to oppose gravity). (ii) mg = BIl => B = (0.2 * 9.8) / (2 * 1.5) = 1.96 / 3 = 0.65 T. (iii) The force will act downwards (with gravity), and the rod will fall with acceleration > g.

### Section E

- (a) Based on torque on a current loop: τ = NIAB. Restoring torque = kθ. Thus, I = (k/NAB)θ. (b) To increase B-field strength and ensure it is radial. (c) Deflection per unit voltage (Φ / V).

- (a) Field of wire 1: B₁ = μ₀I₁ / 2πd. Force on wire 2: F = B₁I₂L. Thus, F/L = μ₀I₁I₂ / 2πd. (b) Standard definition of Ampere.